Power Electronics Lab

Experiment 2 (Part B)

EEE & INST F342

Names and IDs of    Group No.  1    Students	:

| S. No. | ID No. | Name |
| --- | --- | --- |
| 1 | 2018B2A30728H | Muskan Khanna |
| 2 | 2018B2A30764H | Amitesh Badkul |
| 3 | 2018B5A30801H | Manish Patil |

BATCH & TIME    	: P1 (11:00 - 1:00)

File Name: 1_P1_Exp_2

Prepared by

Department of EEE

BITS Pilani, Hyderabad Campus

Date: January 2021

RESULTS

I) R-Load

- Attach the waveforms of a) Input Voltage b) Output Voltage c) Input current and d) Output  Current in Simulink

- Attach the waveforms of a) Diode Voltage b) Diode current for both positive half cycle (Diode D1) and negative half cycle (Diode D3) in Simulink

- Attach the FFT bar chart along with the corresponding waveforms for Input current in Simulink

II) RL-Load

- Attach the waveforms of a) ) Input Voltage b) Output Voltage c) Input current and d) Output  Current in Simulink

- Attach the waveforms of a) Diode Voltage b) Diode current for both positive half cycle (Diode D1) and negative half cycle (Diode D3) in Simulink

- Attach the FFT bar chart along with the corresponding waveforms for Input current in Simulink

- Calculate Performance parameters (Simulink)

|  | R Load | RL load |
| --- | --- | --- |
| VRMS | 23.45 V | 23.48 V |
| IRMS | 0.9381 A | 0.9362 A |
| VAVG | 20.71 V | 20.76 V |
| IAVG | 0.8286 A | 0.8304 A |
| Form factor | 1.13 | 1.131 |
| Ripple Factor | 0.5262 | 0.5283 |
| FFT Analysis |  |  |
| Output Voltage |  |  |
| THD in % | 21.91% | 23.44% |
| VFundamental (RMS) | 0.4221 | 0.4186 |
| 2nd Harmonics (V) (RMS) | 0.0823 | 0.0850 |
| 3rd Harmonics (V) (RMS) | 0.0344 | 0.0377 |
| Input Current |  |  |
| THD in % | 21.85% | 23.26% |
| IFundamental (RMS) | 0.422 | 0.4179 |
| 2nd Harmonics (I) (RMS) | 0.0822 | 0.0844 |
| 3rd Harmonics (I) (RMS) | 0.0343 | 0.0372 |

THD = VAC RMS2-V1RMS2V1RMS2 x 100 %

VAC RMS= VRMS2-VAVG2

-20 log (VFundamental (RMS)) = 7.49 dB

DISCUSSIONS:

There is a little distortion in the input voltage signal . This is due to the fact that we are sending the distorted current into the voltage source . injecting impurities into the source will result in distortion. Hence , the power factor will be low for small devices due to the distorted current. This makes fft analysis crucial and we need it to design filters for our rectifier.

Inductor acts like a current source and as soon as it gets some current then it will start discharging. If values are large enough then the value of current wont come back to zero. In R-L load, the inductor will sustain the value of current leading to continuous current. When this current goes to the source cycle, the source current changes its shape. The advantage of simulation is we can try different values in  simulation to gain different form-factor and ripple-factor.